# SAFE OPERATING PROCEDURE

Tormek Blade Sharpener

| URE OF YOUR MACHINE or TOOL HERE | DO NOT use this machine unless you have had instruction and training in its safe use and operation. ### Teacher permission MUST be given ### to operate this machine. |
| --- | --- |
| The risk of injury when using this machine is MODERATE |  |

## IDENTIFIED RISKS AND HAZARDS

Hazards that may arise when operating the blade sharpener include:

- moving and rotating parts (blades and bits, tool disintegration)

- movement of the workpiece

- electrocution from power faults, faulty equipment or incorrect use

- burns from hot materials or friction

- hand and finger injuries

### PRE-OPERATIONAL SAFETY

- Before using the sharpener, examine the power cord, extension lead, plugs, sockets and power outlet for damage.

- Before starting the sharpener ensure that the wheel, fences, guides and attachments are secure and correctly fitted.

- Always inspect the tool to ensure that there isn’t anything which might damage the wheel or cause injury to the operator.

- Secure and support the tool to be sharpened using guide and appropriate clamps.

## OPERATING SAFETY PRECAUTIONS

- Check the grinding wheel carefully for cracks or damage before switching the machine on.

- Do not operate the sharpening machine with the guide (rail) removed.

- Keep a space of 5mm between the sharpening platform guide (rail) and the grinding wheel.

- After the correct coolant flow has been established, start the sharpening procedure.

- Slide the sharpening holder back and forth on the sharpening platform guide at a speed of about 10 times per minute

- Use the upper surface of the grinding wheel only. Do not use the side surface.